PROJECT DOCUMENT

Smart Scan Strategy for Electronic Warfare

A Machine Learning Based Electronic Support (ES) Receiver Scheduler

Problem Statement ID: 26055

Organization: DRDO  |  Department of Defence Production / IDEX

Category: Software  |  Theme: Robotics and Drones

Prepared as a solution design and demo plan document

# 1. Problem Statement Overview

Problem ID: 26055

Title: Smart Scan strategy for Electronic Warfare

Organization: DRDO, Department of Defence Production / IDEX

Category: Software

Theme: Robotics and Drones

Electronic Warfare (EW) systems must detect hostile communication or radar emitters by scanning a wide frequency spectrum. Because the receiver's instantaneous bandwidth is much smaller than the total spectrum it must cover, it has to sweep across frequency bands over time. Existing approaches rely on pre-mission, open-loop scan schedules that do not adapt to what is actually being observed, and can waste time on non-threatening emitters while missing new or higher-priority ones.

This project develops a closed-loop, machine-learning-based scan scheduler that observes hits and misses in real time, learns which bands are active, adapts when emitter behaviour changes, and decides where the receiver should look next — improving interception rate and reducing intercept time, without relying on prior intelligence about emitters.

## 1.1 Core Objective

- Build a system model of an ES receiver operating against a simulated RF environment with known ground truth.

- Develop figures of merit: probability of detection, probability of false alarm, average intercept rate, average reward/cost, percentage of correct predictions, average intercept time error.

- Design a robust ML-based scheduler that minimizes intercept time and maximizes interception rate.

- Train the model on hits and misses, and outline approaches to optimally intercept periodic-scan and frequency-agile emitters.

# 2. Solution Approach

Signal interception is treated as a two-dimensional, sequential decision-making problem: choosing the correct frequency band at the correct time, under uncertainty and without prior knowledge of emitter behaviour. This maps naturally onto Reinforcement Learning / Multi-Armed Bandit formulations:

| Concept | Maps to |
| --- | --- |
| State | Which band, how recently scanned, recent hit/miss history |
| Action | Which band to scan next |
| Reward | Interception success (hit); penalty for miss or stale coverage |
| Policy | The learned scanning strategy (the “scheduler”) |

The solution is structured in three logical layers:

- RF Environment Simulator — generates ground-truth transmission activity across N frequency bands over time, including fixed, periodic, agile and intermittent emitters.

- Receiver Model — simulates a bandwidth-limited scanner that can observe only the band(s) it chooses to tune to at each time step, returning hit/miss feedback.

- ML Scheduler — the core deliverable; learns from hit/miss history and decides which band to scan next, balancing exploitation of known-active bands with exploration for new or changing activity.

# 3. Operating Scenarios Considered

Because the scheduler must work “in the absence of prior reliable intelligence,” it needs to be validated against every realistic pattern of emitter behaviour, not just the simple case. The following scenarios define the test matrix for the model:

| # | Scenario | What the model must do |
| --- | --- | --- |
| 1 | Stable pattern (fixed hot bands) | Learn and settle on consistently active bands |
| 2 | Pattern switch (hot bands change over time) | Discard stale belief, adapt to the new pattern (recency weighting) |
| 3 | Single dominant band | Allocate most time to it while reserving minimal coverage elsewhere |
| 4 | Multiple equally active bands | Share priority across all of them |
| 5 | Random / no structure | Fall back toward uniform coverage — no false pattern learned |
| 6 | New/surprise emitter appears | Detect via continuous exploration, not miss it |
| 7 | Periodic emitter (fixed interval) | Learn timing and predict next occurrence |
| 8 | Frequency-hopping / agile emitter | Learn the hop sequence or statistics of movement |
| 9 | Intermittent / burst emitter | Avoid mislabeling temporary silence as “dead” |
| 10 | Multiple simultaneous emitters | Prioritize under the receiver's single/limited-channel constraint |
| 11 | Noise / false alarms | Avoid learning from false positives |
| 12 | Activity ≠ threat priority | Allow priority weighting beyond raw activity count, where threat data exists |

# 4. Model Development Strategy (Staged)

The scheduler is built incrementally so each capability can be validated in isolation before the next is layered on — a “crawl, walk, run” approach.

## Stage 1 — Basic Learning

Objective: learn hit/miss statistics per band from repeated scans.

- Environment: fixed pattern, does not change over the run.

- Validates: the model correctly identifies which bands are consistently active (covers Scenarios 1, 3, 4).

## Stage 2 — Adaptation

Objective: respond correctly when the active bands change mid-run.

- Environment: pattern deliberately switched partway through (e.g. first half bands {1,3,5} active, second half {2,4}).

- Mechanism: recency weighting via sliding-window statistics or exponential decay, so old evidence is down-weighted relative to new evidence.

- Validates: how quickly the scheduler switches priority after the environment changes (covers Scenario 2).

## Stage 3 — Exploration Balance

Objective: never let the scheduler become fully greedy — always retain some probability of checking under-sampled bands.

- Environment: an entirely new band activates that was previously silent for the whole run.

- Mechanism: bandit exploration strategy (e.g. ε-greedy, UCB1, or Thompson Sampling).

- Validates: time-to-detect for a first-time emitter (covers Scenarios 6, 9).

## Stage 4 — Advanced / Future Scope

Pattern prediction for periodic and frequency-hopping emitters (Scenarios 7, 8) using sequence-aware models. Planned as a later phase once Stages 1–3 are validated.

# 5. Electronic Support Receiver — Background

The project simulates the behaviour of a bandwidth-limited search/surveillance receiver rather than building or interfacing with physical RF hardware, in line with the problem statement's own direction to use “a simulated RF environment.” For context, real EW receivers fall into the following broad types:

| Receiver Type | Behaviour | Relevance |
| --- | --- | --- |
| Superheterodyne (Swept) | Narrow window swept slowly across the spectrum | Matches the scanning behaviour this project targets |
| Crystal Video Receiver (CVR) | Wideband presence detection, low frequency precision | Fast but imprecise — reference only |
| IFM | Instantaneous frequency measurement of a single signal | High precision, single signal — reference only |
| Channelized Receiver | Parallel fixed channels covering the band | Fast and accurate but hardware-heavy — reference only |
| Wideband Digital Receiver | Digitized spectrum processed in software | Closest analogue to the software scheduler built here |

Within a real EW suite, different receivers serve different roles — search/surveillance (find new emitters), track (stay locked on a known emitter), direction-finding (locate the emitter spatially), and ELINT (deep signal analysis). This project's scope is limited to the search/surveillance scheduling problem, which is what the problem statement asks for.

# 6. System Design

## 6.1 Data Flow

- RF Environment (simulated) holds ground truth: which band is transmitting at each time step.

- Receiver Simulation queries only the band(s) it is tuned to and returns hit/miss (+ optional signal strength).

- Data Logger records band, time step, and result for every query.

- ML Scheduler analyses the log, updates its internal estimate of each band's activity, and selects the next band(s) to scan.

- The decision feeds back to the receiver simulation for the next time step — forming the closed loop the problem statement asks for, as opposed to an open-loop fixed sweep.

## 6.2 What Is Being Built (and What Is Not)

| In scope | Out of scope |
| --- | --- |
| Software simulation of spectrum, receiver and scheduler | Physical RF hardware or real signal capture |
| ML-based scan scheduling logic | Full web application with user accounts/database |
| Metrics and comparison against an open-loop baseline | Multi-receiver / spatial coordination (stretch goal only) |
| A visualization layer to present results | Deep sequence models for hopping prediction (future phase) |

# 7. Evaluation Metrics

| Metric | Definition |
| --- | --- |
| Interception Ratio | % of true transmissions that were successfully scanned/detected |
| Average Intercept Time | Average delay between a signal starting and it being detected |
| Probability of Detection (Pd) | Likelihood that an active emitter is detected when present |
| Probability of False Alarm (Pfa) | Likelihood of registering a detection when none occurred (if noise modeled) |
| Miss Count | Number of transmissions never detected within the run |
| Adaptation Speed | Time steps taken to re-prioritize after a pattern change |
| Reward / Cost Score | Cumulative reward function used to train and benchmark the scheduler |

# 8. Proposed Technology Stack

- Python — core language for simulation and ML

- NumPy — spectrum/time grid and numerical operations

- Gymnasium — custom environment interface (compatible with both bandit and RL approaches)

- Bandit algorithms (ε-greedy, UCB1, Thompson Sampling) — implemented directly for the core scheduler

- Matplotlib / Plotly — waterfall and comparison visualizations

- Stable-Baselines3 (future phase) — for upgrading to full RL (DQN/PPO) once bandit baseline is validated

- Streamlit or a lightweight HTML/JS front-end (optional) — for an interactive demo dashboard

# 9. Development Roadmap

| Phase | Deliverable |
| --- | --- |
| Phase 1 (MVP) | RF simulator + round-robin baseline + bandit scheduler (Stage 1) + core metrics |
| Phase 2 | Adaptation (Stage 2) and exploration balance (Stage 3) added to scheduler |
| Phase 3 | Visualization dashboard for live/side-by-side demo |
| Phase 4 | Periodic-emitter timing prediction; frequency-hopping pattern learning (Stage 4) |
| Phase 5 | Real dataset integration (e.g. Turing synthetic radar dataset) for validation |
| Phase 6 (stretch) | Multi-receiver / spatial coordination |

# 10. Demo Plan

The demo is structured to make the improvement of the smart scheduler over a traditional fixed sweep immediately visible.

## 10.1 Demo Modules

- RF Environment Simulator — configurable number of bands and time steps; supports fixed, switching, and surprise-emitter patterns.

- Baseline Scanner — fixed round-robin sweep, used purely as the comparison point.

- Smart Scanner — the ML scheduler, run through Stages 1–3 against the same environment.

- Metrics & Comparison — interception ratio, average intercept time, miss count, adaptation speed, shown for both scanners side by side.

- Visualization — a waterfall/heatmap (time vs. band) showing ground truth, what was scanned, and what was hit; plus a running comparison chart.

## 10.2 Suggested Default Configuration

In the absence of a specific preference, the following defaults are recommended to keep the demo simple to explain yet clearly illustrative:

- 10 frequency bands, 200–300 time steps.

- Scheduler algorithm: ε-greedy or UCB1 — simple, fast to implement, and easy to explain to evaluators compared to deep RL.

- Scenarios covered in the core demo: stable pattern → pattern switch → surprise emitter (i.e. Stages 1–3), with periodic/hopping emitters flagged as future scope.

- Output form: a Python script producing plots first; an interactive dashboard as a stretch enhancement for presentation.

## 10.3 Success Criteria for the Demo

- Smart scanner shows a materially higher interception ratio than the round-robin baseline on the same environment.

- Smart scanner visibly re-prioritizes within a small number of time steps after a pattern switch.

- Smart scanner still detects a newly-introduced emitter within a bounded number of steps (exploration did not go to zero).

# 11. Future Scope

- Upgrade from multi-armed bandit to full Reinforcement Learning (DQN/PPO) for richer state representation.

- Model periodic and frequency-hopping emitters explicitly using sequence prediction.

- Incorporate false-alarm/noise modeling for a realistic Pfa metric.

- Validate against public/synthetic radar datasets (e.g. Turing synthetic radar dataset) in addition to the custom simulator.

- Extend to multi-receiver and spatially-distributed scanning coordination.

End of Document